# Pre-Characterization Process Risk Assessment

A-Mab Drug Substance — Risk Ranking and Study-Type Assignment — RA-001

Manufacturing Science & Technology (MSAT) / Quality Risk Management

2026-07-24

Table of contents

| Field | Value |
| --- | --- |
| Document ID | RA-001 |
| Document class | Pre-Characterization Process Risk Assessment |
| Title | Pre-Characterization Process Risk Assessment — A-Mab Drug Substance |
| Product | A-Mab |
| Version | 1.0 |
| Effective date | 2026-07-24 |
| Status | Approved (synthetic) |
| Document owner | Manufacturing Science & Technology (MSAT) |
| Sponsor / sites | Novacyte Biologics; Novacyte Biologics — Cambridge, MA (Development) → Novacyte Biologics — Grafton, WI (Commercial DS) |

SYNTHETIC DOCUMENT — NOT A REAL RECORD. Illustrative content generated from a seeded model of the A-Mab case study (CMC Biotech Working Group, 2009). Novacyte Biologics and all sites, SOP/AMV numbers and signatories are fictional. For NLP corpus development only; not for any regulated use.

## Approvals

Table 1: Approval record (synthetic; signatures not applied). Role; Function; Name (synthetic); Signature; Date. Author; Quality Risk Management / MSAT; —; —; —. Reviewer; Process Development; —; —; —. Reviewer; Analytical Development; —; —; —. Reviewer; Manufacturing Science; —; —; —. Approver; Quality Assurance; —; —; —.

## Abbreviations

CPP, critical process parameter · CQA, critical quality attribute · D, detection · DoE, design of experiments · DS, drug substance · FMEA, failure mode and effects analysis · GPP, general process parameter · ICH, International Council for Harmonisation · KPP, key process parameter · MSAT, Manufacturing Science and Technology · O, occurrence · PC, process characterization · RPN, risk priority number · RRF, risk ranking and filtering · S, severity · WC-CPP, well-controlled CPP.

# 1. Purpose and scope

This Pre-Characterization Process Risk Assessment (RA-001) prioritizes the A-Mab drug-substance process parameters for characterization and assigns each parameter a study type, before the characterization studies are executed. It is the first (pre-hoc) risk assessment of the A-Mab lifecycle of iterative risk assessments: it uses prior knowledge and early development experience — not the results of the characterization studies — to decide which parameters carry a credible risk to a critical quality attribute (CQA) and therefore warrant a multivariate designed experiment, which warrant a univariate study, and which require only routine control. Its output is the characterization scope that is handed to each per-unit-operation Process Characterization Plan (PCP-003 … PCP-010).

Scope. The 37 process parameters of the drug-substance train (production bioreactor through final UF/DF), assessed for their prospective (pre-characterization) risk of impacting the 10 drug-substance CQAs and for the resulting study-type assignment. This assessment does not classify parameters as CPP/WC-CPP/KPP/GPP — that is an output of the characterization studies, reported in each PCR and consolidated in the Process Characterization Master Report (PCMR-001); nor does it use the post-characterization residual risk.

Basis. The assessment is derived from the Process Transfer Plan (PTP-001) product and process description and follows the A-Mab quality-risk-management methodology (CMC Biotech Working Group 2009) — the two-axis CQA criticality tools and the risk-ranking-and-filtering (RRF) study-type rule — within the ICH Q9 quality-risk-management framework (International Council for Harmonisation 2023) and the QbD approach of ICH Q8 and Q11 (International Council for Harmonisation 2009, 2012). The curated per-parameter failure-mode and effect content is the same content source used to build the post-characterization process FMEA.

# 2. Related documents

This assessment is a parent risk document for the characterization package; the full package is cross-referenced for traceability.

Table 2: The A-Mab drug-substance document package referenced by this risk assessment. Document ID; Document class; Subject. PTP-001; Process Transfer Plan; A-Mab Drug Substance. PCMP-001; Process Characterization Master Plan; A-Mab Drug Substance. PCP-003; Process Characterization Plan; Production Bioreactor (Step 3). PCR-003; Process Characterization Report; Production Bioreactor (Step 3). PCP-004; Process Characterization Plan; Harvest and Clarification (Step 4). PCR-004; Process Characterization Report; Harvest and Clarification (Step 4). PCP-005; Process Characterization Plan; Protein A Chromatography (Step 5). PCR-005; Process Characterization Report; Protein A Chromatography (Step 5). PCP-006; Process Characterization Plan; Low-pH Viral Inactivation (Step 6). PCR-006; Process Characterization Report; Low-pH Viral Inactivation (Step 6). PCP-007; Process Characterization Plan; Cation Exchange Chromatography (Step 7). PCR-007; Process Characterization Report; Cation Exchange Chromatography (Step 7). PCP-008; Process Characterization Plan; Anion Exchange Chromatography (Step 8). PCR-008; Process Characterization Report; Anion Exchange Chromatography (Step 8). PCP-009; Process Characterization Plan; Small-Virus Retentive Filtration (Step 9). PCR-009; Process Characterization Report; Small-Virus Retentive Filtration (Step 9). PCP-010; Process Characterization Plan; Ultrafiltration / Diafiltration (Step 10). PCR-010; Process Characterization Report; Ultrafiltration / Diafiltration (Step 10). PCMR-001; Process Characterization Master Report; A-Mab Drug Substance.

# 3. Risk-assessment methodology

## 3.1 Two distinct criticality assessments

Consistent with the A-Mab case study, two distinct assessments are kept separate (CMC Biotech Working Group 2009). The CQA criticality (quality-attribute criticality) is scored on two axes only — Tool #1: Risk Score = Impact × Uncertainty, and Tool #2: Severity × Likelihood — explicitly excluding manufacturing capability and detectability; this yields a continuum of criticality, not a binary critical / non-critical split. The process-parameter risk (this assessment) then asks, for each parameter, whether its variability could impact a CQA and how the parameter should therefore be studied. Criticality is treated as a continuum throughout (ICH Q9) (International Council for Harmonisation 2023).

## 3.2 Pre-characterization risk ranking and study-type assignment

Because the characterization studies have not yet been executed, this assessment uses the initial (pre-characterization) risk: a parameter with a credible impact to a CQA has an uncertain effect and poor in-process detectability (the effect is often detectable only at a downstream step or at release), so its initial risk is high and it is prioritized for study. A risk-ranking-and-filtering (RRF) screen scores each parameter’s potential main-effect and interaction impact and assigns the study type accordingly: a parameter with a credible main-effect and interaction risk to a CQA is carried into the unit operation’s multivariate designed experiment (DoE); a parameter with a credible CQA impact but no expected interaction is studied by a justified univariate study; and a parameter with no credible CQA impact — one that affects only process performance — is studied univariately for its performance effect (CMC Biotech Working Group 2009). The classification of each parameter (CPP / WC-CPP / KPP / GPP) is deliberately not assigned here; it is an output of the executed studies.

## 3.3 Prospective severity and initial risk

The prospective severity of each parameter is taken as the maximum severity of the CQA(s) it could impact, on the A-Mab severity scale (2–10). The initial risk priority number (RPN = Severity × Occurrence × Detection) uses the pre-characterization occurrence and detection: an uncertain effect (higher occurrence) and poor pre-characterization detectability (highest for viral-clearance attributes, which cannot be detected by in-process testing). The initial RPN is used only to rank and prioritize; it is expected to fall after characterization and the resulting control strategy, as shown in the post-characterization process FMEA.

# 4. CQA criticality framework

The CQAs in scope and their criticality (Tool #1 = Impact × Uncertainty; Tool #2 severity) are given in Table 3. Attributes rated High (H) or Very High (VH) are treated as Critical, and their associated parameters carry the highest prospective severity.

Table 3: Drug-substance CQA criticality framework (Tool #1 = Impact × Uncertainty; Tool #2 = severity). CQA; Category; Acceptance; Criticality; Tool #1; Tool #2 sev.. Afucosylation; glycosylation; 2–13 % of glycans; H; 60; 7. Galactosylation (total %Gal); glycosylation; 10–40 % (G1+G2 equiv.); H; 48; 7. High Mannose; glycosylation; 3–10 % of glycans; VH; 80; 7. Aggregates (HMW); size variant; 0–5 % HMW (SEC); H; 60; 5. Acidic charge variants (deamidation); charge variant; 18–40 % (CEX/icIEF); VL; 4; 3. Host Cell Protein (HCP); process impurity; 0–100 ng/mg; M-H; 36; 5. Residual DNA; process impurity; 0–0.001 ng/dose; VL; 6; 1. Leached Protein A; process impurity; 0–5 ppm; L-M; 16; 5. Viral clearance — XMuLV (enveloped); viral safety; 16.7–30 log10 (cumulative); VH; 20; 9. Viral clearance — MVM (parvovirus); viral safety; 8.6–20 log10 (cumulative); VH; 20; 9.

# 5. Process-parameter risk ranking and study-type assignment

The prospective risk ranking of every process parameter, with the CQA(s) at risk, the prospective severity, the initial (pre-characterization) RPN, the assigned study type and the characterization priority, is given in Table 4. Parameters that could impact a CQA are prioritized for study; parameters that affect only process performance are ranked Low (product-quality) priority and studied univariately for their performance effect. The prospective failure mode and effect underlying each ranking are tabulated in Appendix A.

Table 4: Pre-characterization process-parameter risk ranking and study-type assignment (initial RPN = prospective Severity × pre-characterization Occurrence × Detection). Step; Unit operation; Parameter; CQA(s) at risk; Sev.; Init. RPN; Study type; Priority. 3; Production Bioreactor; Culture pH; Afucosylation, Galactosylation (total %Gal), High Mannose, Acidic charge variants (deamidation), Aggregates (HMW); 10; 560; multivariate DoE; High. 3; Production Bioreactor; Culture temperature; Afucosylation, Galactosylation (total %Gal), Acidic charge variants (deamidation); 8; 448; multivariate DoE; High. 3; Production Bioreactor; Dissolved CO2 (pCO2); Galactosylation (total %Gal), Acidic charge variants (deamidation); 8; 448; multivariate DoE; High. 3; Production Bioreactor; Osmolality; Galactosylation (total %Gal), Acidic charge variants (deamidation); 8; 448; multivariate DoE; High. 3; Production Bioreactor; Culture duration; Afucosylation, Galactosylation (total %Gal), Aggregates (HMW), Host Cell Protein (HCP), Residual DNA; 8; 448; multivariate DoE; High. 3; Production Bioreactor; Dissolved oxygen; process performance; 4; 96; univariate; Low. 3; Production Bioreactor; Initial viable cell conc.; process performance; 4; 96; univariate; Low. 3; Production Bioreactor; Basal medium concentration; process performance; 4; 96; univariate; Low. 3; Production Bioreactor; Nutrient feed-1 volume; process performance; 4; 96; univariate; Low. 4; Harvest and Clarification; Centrifugation (rcf); process performance; 4; 96; univariate; Low. 4; Harvest and Clarification; Depth filter load; process performance; 4; 96; univariate; Low. 4; Harvest and Clarification; Post-clarification turbidity; process performance; 4; 96; univariate; Low. 5; Protein A Chromatography; Protein load; Host Cell Protein (HCP); 6; 336; multivariate DoE; Medium. 5; Protein A Chromatography; Elution buffer pH; Host Cell Protein (HCP); 6; 336; multivariate DoE; Medium. 5; Protein A Chromatography; Load flow rate; process performance; 4; 96; multivariate DoE; Low. 5; Protein A Chromatography; End of pool collect; process performance; 4; 96; multivariate DoE; Low. 5; Protein A Chromatography; Operating temperature; process performance; 4; 96; univariate; Low. 5; Protein A Chromatography; Bed height; process performance; 4; 96; univariate; Low. 6; Low-pH Viral Inactivation; Inactivation pH; Viral clearance — XMuLV (enveloped), Aggregates (HMW); 10; 700; multivariate DoE; High. 6; Low-pH Viral Inactivation; Hold time; Aggregates (HMW), Viral clearance — XMuLV (enveloped); 10; 700; multivariate DoE; High. 6; Low-pH Viral Inactivation; Temperature; Viral clearance — XMuLV (enveloped); 10; 700; multivariate DoE; High. 6; Low-pH Viral Inactivation; A-Mab concentration; process performance; 4; 96; univariate; Low. 7; Cation Exchange Chromatography; Protein load; Aggregates (HMW), Host Cell Protein (HCP); 8; 448; multivariate DoE; High. 7; Cation Exchange Chromatography; Load/Wash conductivity; Host Cell Protein (HCP); 6; 336; multivariate DoE; Medium. 7; Cation Exchange Chromatography; Elution buffer pH; Aggregates (HMW); 8; 448; multivariate DoE; High. 7; Cation Exchange Chromatography; Elution stop collect; Aggregates (HMW); 8; 448; multivariate DoE; High. 7; Cation Exchange Chromatography; Elution flow rate; process performance; 4; 96; univariate; Low. 8; Anion Exchange Chromatography; Load pH; Host Cell Protein (HCP), Viral clearance — XMuLV (enveloped), Viral clearance — MVM (parvovirus); 10; 700; multivariate DoE; High. 8; Anion Exchange Chromatography; Equil/Wash-1 conductivity; Host Cell Protein (HCP); 6; 336; multivariate DoE; Medium. 8; Anion Exchange Chromatography; Load conductivity; Viral clearance — XMuLV (enveloped), Viral clearance — MVM (parvovirus); 10; 700; multivariate DoE; High. 8; Anion Exchange Chromatography; Protein load; Host Cell Protein (HCP); 6; 336; multivariate DoE; Medium. 8; Anion Exchange Chromatography; Operating flow rate; Viral clearance — XMuLV (enveloped), Viral clearance — MVM (parvovirus); 10; 700; univariate (justified); High. 9; Small-Virus Retentive Filtration; Filtration volume (load); Viral clearance — MVM (parvovirus); 10; 700; multivariate DoE; High. 9; Small-Virus Retentive Filtration; Filtration pressure; Viral clearance — MVM (parvovirus), Viral clearance — XMuLV (enveloped); 10; 700; multivariate DoE; High. 10; Ultrafiltration / Diafiltration; Number of diavolumes; process performance; 4; 96; univariate; Low. 10; Ultrafiltration / Diafiltration; Transmembrane pressure; process performance; 4; 96; univariate; Low. 10; Ultrafiltration / Diafiltration; Final DS concentration; process performance; 4; 96; univariate; Low.

Of the 37 parameters, 21 carry a credible prospective risk to a CQA and 16 affect only process performance. 16 are ranked High priority (a CQA of severity ≥ 8 at risk), 5 Medium, and 16 Low.

# 6. Characterization scope

The study-type assignment defines the characterization scope handed to the per-unit-operation plans (Table 5): 22 parameters are assigned to a multivariate DoE, 1 to a justified univariate study, and 15 to univariate ranging. Multivariate designed experiments are therefore planned for 6 unit operations (Steps 3, 5, 6, 7, 8, 9); the remaining steps — harvest / clarification and final UF/DF — carry no credible CQA risk and are characterized univariately.

Table 5: Characterization scope handed to the Process Characterization Plans (PCP-003 … PCP-010). Assigned study type; Parameters; RRF basis. Multivariate DoE; 22; Credible main-effect and interaction risk to a CQA. Univariate (justified); 1; Credible CQA risk without an expected interaction. Univariate; 15; No credible CQA risk; process-performance study only.

This scope is carried into each Process Characterization Plan, which specifies the exact designs, ranges and acceptance criteria; the executed studies then classify each parameter and define the design spaces and control strategy (reported in the PCRs and consolidated in PCMR-001 and the post-characterization process FMEA).

# 7. Roles and responsibilities

Table 6: Roles and responsibilities. Function; Responsibility. Quality Risk Management / MSAT; Author the assessment; run the RRF screen; assign study types. Process Development; Provide prior-knowledge risk input; execute the assigned studies under the PCPs. Analytical Development; Confirm the CQA criticality and the detectability inputs. Quality Assurance; Approve the assessment and the resulting characterization scope.

# 8. Approvals

Approval of this assessment authorizes the characterization scope and study-type assignments carried into the Process Characterization Plans. Signatures are recorded in Table 1.

# References

CMC Biotech Working Group. 2009. A-Mab: A Case Study in Bioprocess Development. CASSS.

International Council for Harmonisation. 2009. ICH Q8(R2): Pharmaceutical Development. ICH.

International Council for Harmonisation. 2012. ICH Q11: Development and Manufacture of Drug Substances. ICH.

International Council for Harmonisation. 2023. ICH Q9(R1): Quality Risk Management. ICH.

# Appendix A — Prospective failure modes and effects

The prospective (pre-characterization) failure mode and effect underlying each parameter’s risk ranking, from the curated A-Mab failure-mode content source. These are the hypotheses the characterization studies test; the confirmed effects and the resulting classification are reported in the PCRs and the post-characterization process FMEA.

Table 7: Appendix A — prospective failure modes and effects per process parameter. Step; Parameter; Potential failure mode; Potential effect; CQA(s) at risk. 3; Culture pH; pH excursion outside the characterized range (6.6-7.1); Shifts glycosylation (ADCC via afucosylation; CDC via galactosylation) and charge/aggregate profile; Afucosylation, Galactosylation (total %Gal), High Mannose, Acidic charge variants (deamidation), Aggregates (HMW). 3; Culture temperature; Temperature excursion outside 34-36 C; Alters afucosylation, galactosylation and acidic charge variants; interacts with pH; Afucosylation, Galactosylation (total %Gal), Acidic charge variants (deamidation). 3; Dissolved CO2 (pCO2); Dissolved CO2 outside 40-160 mmHg; Strong effect on galactosylation and acidic variants; Galactosylation (total %Gal), Acidic charge variants (deamidation). 3; Osmolality; Osmolality outside 360-440 mOsm; Affects galactosylation and acidic variants; Galactosylation (total %Gal), Acidic charge variants (deamidation). 3; Culture duration; Culture extended beyond 15-19 days / harvested on low viability; Lowers afucosylation & galactosylation; raises HCP/DNA and aggregate as viability declines; Afucosylation, Galactosylation (total %Gal), Aggregates (HMW), Host Cell Protein (HCP), Residual DNA. 3; Dissolved oxygen; DO outside 30-70%; Impacts peak VCD, titer and process consistency (no significant CQA impact); process performance. 3; Initial viable cell conc.; Inoculation density outside 0.5-1.5e6/mL; Drives peak/integral VCD and titer; no product-quality impact; process performance. 3; Basal medium concentration; Basal medium concentration outside 0.8-1.6X; Statistically significant but shallow effect on afucosylation; drives titer; process performance. 3; Nutrient feed-1 volume; Nutrient feed-1 volume outside 9-15% WV; Slight effect on galactosylation; critical for titer; process performance. 4; Centrifugation (rcf); Centrifugation rcf outside 6000-12000 g; Affects clarification/yield (no CQA impact); process performance. 4; Depth filter load; Depth filter load outside 80-160 L/m2; Affects filter capacity/turbidity (no CQA impact); process performance. 4; Post-clarification turbidity; Post-clarification turbidity high; Feed quality to Protein A (no CQA impact); process performance. 5; Protein load; Protein load above characterized range (>50 g/L resin); Increases pool HCP (interaction with elution pH); Host Cell Protein (HCP). 5; Elution buffer pH; Elution pH below characterized range (<3.2); Increases pool HCP; also sets up viral inactivation feed; Host Cell Protein (HCP). 5; Load flow rate; Load flow rate high with high load; Reduces product yield (no CQA impact); process performance. 5; End of pool collect; End-of-pool collection outside 2.0-3.2 CV; Affects product yield (no CQA impact); process performance. 5; Operating temperature; Parameter outside characterized range; No significant product-quality impact expected; process performance. 5; Bed height; Parameter outside characterized range; No significant product-quality impact expected; process performance. 6; Inactivation pH; pH > 4.0 (incomplete inactivation) or pH < 3.2 (aggregation/precipitation); Loss of XMuLV inactivation assurance or increased aggregate; Viral clearance — XMuLV (enveloped), Aggregates (HMW). 6; Hold time; Hold time < 60 min (incomplete) or > 180 min (aggregation); Incomplete inactivation or increased aggregate; Aggregates (HMW), Viral clearance — XMuLV (enveloped). 6; Temperature; Temperature < 15 C; Lower inactivation rate at short time / higher pH; Viral clearance — XMuLV (enveloped). 6; A-Mab concentration; Concentration > 35 g/L; No significant effect on inactivation, aggregation or charge variants; process performance. 7; Protein load; Protein load above characterized range (>40 g/L resin); Reduced aggregate and HCP clearance (interactions with pH and wash conductivity); Aggregates (HMW), Host Cell Protein (HCP). 7; Load/Wash conductivity; Load/wash conductivity below 3 mS/cm; Reduced HCP clearance; Host Cell Protein (HCP). 7; Elution buffer pH; Elution pH outside 5.8-6.2; Alters aggregate distribution and yield; Aggregates (HMW). 7; Elution stop collect; Extended elution stop-collect (>1.5 OD); Minor increase in pool aggregate; Aggregates (HMW). 7; Elution flow rate; Elution flow rate outside 100-300 cm/hr; Affects peak shape and yield (minor aggregate); process performance. 8; Load pH; Load pH below characterized range (<7.2); Reduced HCP removal and reduced XMuLV/MVM clearance; Host Cell Protein (HCP), Viral clearance — XMuLV (enveloped), Viral clearance — MVM (parvovirus). 8; Equil/Wash-1 conductivity; Equil/Wash-1 conductivity above 3.6 mS/cm; Reduced HCP removal; Host Cell Protein (HCP). 8; Load conductivity; Load conductivity above characterized range; Reduced viral clearance (interacts with pH); Viral clearance — XMuLV (enveloped), Viral clearance — MVM (parvovirus). 8; Protein load; Protein load above characterized range (>300 g/L resin); Reduced HCP removal capacity (viral clearance robust); Host Cell Protein (HCP). 8; Operating flow rate; Operating flow rate above 450 cm/hr; Reduced residence time / viral clearance margin; Viral clearance — XMuLV (enveloped), Viral clearance — MVM (parvovirus). 9; Filtration volume (load); Volumetric load above 105 L/m2; Reduced MVM log-reduction (parvovirus breakthrough risk); Viral clearance — MVM (parvovirus). 9; Filtration pressure; Pressure outside filter operating limits; Potential impact on virus removal at high load; Viral clearance — MVM (parvovirus), Viral clearance — XMuLV (enveloped). 10; Number of diavolumes; Diavolumes < 7; Incomplete buffer exchange (formulation; not part of DS characterization); process performance. 10; Transmembrane pressure; TMP outside 10-15 psi; Affects flux/processing (formulation); process performance. 10; Final DS concentration; Final concentration outside 65-85 g/L; DS concentration spec (formulation); process performance.